Flow Control

SystemVerilog Traffic Scheduling & Shaping Engine
User Manual

Version 1.0
2026-04-18

# Table of Contents

1. Project Overview

2. Project Structure

3. Quick Start

4. Core Concepts

4.1 Traffic Models

4.2 Traffic Queue

4.3 Scheduler

4.4 Port Shaper

4.5 Traffic Monitor

4.6 Flow Controller

5. API Reference

5.1 Enumerations

5.2 Structures

5.3 flow_controller Methods

6. Usage Examples

6.1 Basic Rate-Limited Traffic

6.2 Token Bucket with Burst

6.3 Multi-Queue WRR Scheduling

6.4 Strict Priority Scheduling

6.5 Mixed SP+WRR Scheduling

6.6 Burst Traffic Pattern

6.7 Random Traffic Model

6.8 Step Rate Profile

6.9 Custom Traffic Model

6.10 Statistics & Monitoring

6.11 Complete Test Bench

7. Test Suite

8. Building & Running

# 1. Project Overview

Flow Control is a SystemVerilog-based traffic scheduling and shaping engine designed for simulation with Synopsys VCS. It provides a complete framework for generating, scheduling, shaping, and monitoring network traffic at the packet level.

Key features:

Five built-in traffic models: constant rate, token bucket, burst, random, and step profile

Three scheduling algorithms: Strict Priority (SP), Weighted Round Robin (WRR), and Mixed SP+WRR

Port-level token bucket rate limiter

Real-time statistics collection with windowed rate monitoring

Extensible via custom traffic models

Comprehensive test suite with 89 test cases

# 2. Project Structure

flow_control/

+-- Makefile                        # Build system

+-- filelist.f                      # VCS compilation file list

+-- src/

|   +-- common/

|   |   +-- flow_defines.sv         # Type definitions and enums

|   +-- models/

|   |   +-- traffic_model_base.sv   # Abstract base class

|   |   +-- rate_model.sv           # Constant rate model

|   |   +-- token_bucket_model.sv   # Rate + burst model

|   |   +-- burst_model.sv          # Bursty traffic model

|   |   +-- random_model.sv         # Randomized inter-packet gaps

|   |   +-- step_model.sv           # Time-segmented rate profiles

|   +-- core/

|   |   +-- traffic_queue.sv        # Per-queue FIFO container

|   |   +-- scheduler.sv            # Queue selection (SP/WRR/Mixed)

|   |   +-- port_shaper.sv          # Token bucket port rate limiter

|   |   +-- flow_controller.sv      # Top-level orchestrator

|   +-- monitor/

|       +-- traffic_monitor.sv      # Statistics & windowed rate check

+-- test/

+-- test_traffic_models.sv      # Traffic model unit tests

+-- test_queue.sv               # Queue FIFO tests

+-- test_scheduler.sv           # Scheduling algorithm tests

+-- test_port_shaper.sv         # Token bucket tests

+-- test_monitor.sv             # Statistics collection tests

+-- test_flow_controller.sv     # Integration tests

## Architecture Diagram

Input Packets --> Queue[N] + Model --> Scheduler --> Port Shaper --> Output

|                                            |

Traffic Monitor <-------- record() --------  Statistics

Per-Queue Control:

- Independent traffic model (rate, burst, random, step, or custom)

- Priority or weight-based scheduling

- Real-time statistics collection

Global Control:

- Total port rate limit via token bucket

- Multiple scheduling modes (SP, WRR, hybrid)

- Configurable duration or packet count limits

# 3. Quick Start

The following minimal example creates a flow controller with one queue at 1 Gbps, sends 100 packets of 1000 bytes each, and prints a statistics report.

program quick_start;

`include "core/flow_controller.sv"

initial begin

flow_controller fc = new();

byte unsigned pkt[$];

int qid;

byte unsigned out_data[$];

realtime send_time;

// 1. Configure

fc.set_port_rate(10000.0);          // 10 Gbps port

fc.set_duration(100us);             // Run for 100 us

// 2. Add queue with rate model at 1 Gbps

fc.add_queue(0, MODEL_RATE);

fc.set_queue_rate(0, 1000.0);       // 1 Gbps

// 3. Generate packets

pkt = {};

for (int i = 0; i < 1000; i++) pkt.push_back(i % 256);

for (int i = 0; i < 100; i++)

fc.push_packet(0, pkt);

// 4. Schedule and send

fc.start_schedule();

while (fc.get_next_scheduled(qid, out_data, send_time))

;  // Process out_data if needed

// 5. Print report

fc.report();

end

endprogram

Compile and run:

# Set VCS environment

export VCS_HOME=/opt/synopsys/vcs/Q-2020.03-SP2-7

export PATH=$VCS_HOME/bin:$PATH

# Compile and simulate

vcs -full64 -sverilog -timescale=1ns/1ps \

-f filelist.f +incdir+src quick_start.sv -o simv

./simv

# 4. Core Concepts

## 4.1 Traffic Models

Traffic models control the inter-packet gap (IPG) for each queue. All models inherit from the abstract base class traffic_model_base and implement the get_interval(pkt_size, current_time) method, which returns the time to wait before the next packet.

| Model | Type Enum | Description | Key Parameters |
| --- | --- | --- | --- |
| rate_model | MODEL_RATE | Constant rate, evenly-spaced packets | rate_mbps |
| token_bucket_model | MODEL_TOKEN_BUCKET | Rate with burst allowance | rate_mbps, burst_size_bytes |
| burst_model | MODEL_BURST | Send N packets, then pause | send_count, pause_time |
| random_model | MODEL_RANDOM | Stochastic IPG | avg_rate_mbps, dist_type |
| step_model | MODEL_STEP | Time-segmented rate profile | steps[$], loop_enable |
| (custom) | MODEL_CUSTOM | User-defined model | (user-defined) |

### rate_model

Generates evenly-spaced packets at a constant rate. The inter-packet gap is calculated as: gap = (pkt_size * 8) / (rate_mbps * 1e6) seconds.

### token_bucket_model

Implements a token bucket algorithm. Tokens refill at the configured rate, capped at burst_size_bytes. If sufficient tokens are available, the packet is sent immediately (gap = 0). Otherwise, the gap equals the time needed to refill enough tokens for the packet.

### burst_model

Sends send_count packets with zero gap (back-to-back burst), then waits pause_time before the next burst. Useful for simulating bursty traffic patterns.

### random_model

Generates random inter-packet gaps with three distribution options:

| Distribution | Enum | Description |
| --- | --- | --- |
| Uniform | DIST_UNIFORM | Gap uniformly distributed in [0.5*mean, 1.5*mean] |
| Poisson | DIST_POISSON | Exponential distribution (Poisson process) |
| Normal | DIST_NORMAL | Gaussian with sigma = 0.2 * mean |

### step_model

Supports time-segmented rate profiles. Each step has a duration and rate. Steps are traversed sequentially. When loop_enable is set, the profile repeats.

## 4.2 Traffic Queue

traffic_queue is a per-queue FIFO container. Each queue has:

queue_id: Unique identifier

model: Associated traffic model (determines IPG)

prio: Priority level (higher = more priority, used by SP scheduler)

weight: Weight (used by WRR scheduler, higher = more bandwidth share)

Internal FIFO: Stores packets as dynamic byte arrays

## 4.3 Scheduler

The scheduler selects which queue sends the next packet. Three modes:

### Strict Priority (SP)

Always selects the queue with the highest prio value that has packets. Lower-priority queues are only served when all higher-priority queues are empty.

### Weighted Round Robin (WRR)

Each queue gets bandwidth proportional to its weight. Uses a Deficit Round Robin (DRR) algorithm with quantum = 1500 bytes. Each queue receives quantum * weight bytes of credit per round. The queue is served until its credit is depleted, then the next queue is served.

Example: With weight 3:1, queue 0 gets ~75% bandwidth and queue 1 gets ~25%.

### Mixed SP+WRR

Queues with prio > 0 are served first using Strict Priority. Queues with prio = 0 are served using WRR among themselves.

## 4.4 Port Shaper

The port_shaper applies a token bucket rate limiter at the port level, limiting the total output bandwidth across all queues.

| Parameter | Default | Description |
| --- | --- | --- |
| port_rate_mbps | 10000.0 | Maximum port rate in Mbps |
| burst_size_bytes | 16000 | Maximum burst size in bytes |

## 4.5 Traffic Monitor

The traffic_monitor collects per-queue statistics and performs windowed rate checking.

| Metric | Description |
| --- | --- |
| total_bytes | Total bytes transmitted |
| total_packets | Total packets transmitted |
| actual_rate_mbps | Measured transmission rate |
| configured_rate_mbps | Expected rate |
| deviation_pct | Percentage deviation from expected rate |
| burst_max_bytes | Maximum bytes in any window |
| window_violations | Windows where rate exceeded tolerance |

The monitor uses a sliding window (default 10 us) to check whether each queue's instantaneous rate stays within the configured tolerance (default 5%).

## 4.6 Flow Controller

flow_controller is the top-level orchestrator that integrates all components. Typical workflow:

1. Create: flow_controller fc = new();

2. Configure port: set_port_rate(), set_duration()

3. Add queues: add_queue(), set_queue_rate(), set_queue_weight()

4. Set scheduler: set_scheduler_mode()

5. Push packets: push_packet(queue_id, data)

6. Start: start_schedule()

7. Retrieve: while (get_next_scheduled(...)) process packets

8. Report: report() or get_queue_stats()

# 5. API Reference

## 5.1 Enumerations

### traffic_model_type_e

| Value | Description |
| --- | --- |
| MODEL_RATE | Constant rate model |
| MODEL_TOKEN_BUCKET | Token bucket model (rate + burst) |
| MODEL_BURST | Burst traffic model |
| MODEL_RANDOM | Random distribution model |
| MODEL_STEP | Step rate profile model |
| MODEL_CUSTOM | User-defined custom model |

### scheduler_mode_e

| Value | Description |
| --- | --- |
| SCHED_STRICT_PRIORITY | Strict Priority scheduling |
| SCHED_WRR | Weighted Round Robin scheduling |
| SCHED_SP_WRR_MIXED | Mixed SP + WRR scheduling |

### distribution_e

| Value | Description |
| --- | --- |
| DIST_UNIFORM | Uniform distribution |
| DIST_POISSON | Poisson (exponential) distribution |
| DIST_NORMAL | Normal (Gaussian) distribution |

## 5.2 Structures

### step_cfg_t

| Field | Type | Description |
| --- | --- | --- |
| duration | realtime | Duration of this step |
| rate_mbps | real | Rate in Mbps for this step |

### queue_stats_t

| Field | Type | Description |
| --- | --- | --- |
| configured_rate_mbps | real | Expected rate in Mbps |
| actual_rate_mbps | real | Measured actual rate in Mbps |
| deviation_pct | real | Deviation percentage |
| total_bytes | longint | Total bytes transmitted |
| total_packets | longint | Total packets transmitted |
| burst_max_bytes | longint | Maximum bytes in any window |
| window_violations | int | Number of window violations |

## 5.3 flow_controller Methods

### Configuration

| Method | Parameters | Description |
| --- | --- | --- |
| set_port_rate() | real rate_mbps | Set total port bandwidth (Mbps) |
| set_port_burst() | int burst_bytes | Set port burst capacity (bytes) |
| set_duration() | realtime dur | Set scheduling duration |
| set_time_unit() | realtime unit | Set time unit (default 1us) |
| set_debug_level() | int level | Set debug verbosity (0/1/2) |
| set_scheduler_mode() | scheduler_mode_e mode | Set scheduling algorithm |

### Queue Management

| Method | Parameters | Description |
| --- | --- | --- |
| add_queue() | int queue_id, traffic_model_type_e type | Create queue with model |
| set_queue_rate() | int queue_id, real rate_mbps | Set queue rate |
| set_queue_burst() | int queue_id, int burst_bytes | Set token bucket burst |
| set_queue_priority() | int queue_id, int prio | Set queue priority (for SP) |
| set_queue_weight() | int queue_id, int weight | Set queue weight (for WRR) |
| set_burst_param() | int qid, int count, realtime pause | Configure burst model |
| set_random_param() | int qid, real rate, distribution_e dist | Configure random model |
| set_step_param() | int qid, step_cfg_t steps[$] | Configure step model |
| set_queue_model() | int qid, traffic_model_base model | Set custom model |
| push_packet() | int qid, byte unsigned data[$] | Enqueue packet |

### Execution

| Method | Parameters | Returns | Description |
| --- | --- | --- | --- |
| start_schedule() | int pkt_count=-1 | void | Start scheduling (-1=duration mode) |
| get_next_scheduled() | out int qid, out byte[] data, out realtime time | bit | Get next packet (0=done) |
| is_running() |  | bit | Check if scheduler is active |

### Monitoring

| Method | Parameters | Returns | Description |
| --- | --- | --- | --- |
| report() |  | void | Print statistics report |
| get_queue_stats() | int queue_id | queue_stats_t | Get statistics for queue |
| monitor_packet() | int qid, byte[] data, realtime time | void | Record received packet |

# 6. Usage Examples

## 6.1 Basic Rate-Limited Traffic

Send constant-rate traffic from a single queue at 1 Gbps through a 10 Gbps port.

flow_controller fc = new();

byte unsigned pkt[$];

int qid;

byte unsigned out_data[$];

realtime send_time;

// Configure port and duration

fc.set_port_rate(10000.0);              // 10 Gbps port

fc.set_duration(100us);

// Add one queue at 1 Gbps

fc.add_queue(0, MODEL_RATE);

fc.set_queue_rate(0, 1000.0);           // 1 Gbps

// Generate 100 packets of 1500 bytes

pkt = {};

for (int i = 0; i < 1500; i++) pkt.push_back(i % 256);

for (int i = 0; i < 100; i++)

fc.push_packet(0, pkt);

// Run

fc.start_schedule();

while (fc.get_next_scheduled(qid, out_data, send_time)) begin

$display("[%0t] Sent %0d bytes from queue %0d", send_time, out_data.size(), qid);

end

fc.report();

## 6.2 Token Bucket with Burst

Token bucket allows burst transmission up to burst size, then rate-limits.

fc.add_queue(0, MODEL_TOKEN_BUCKET);

fc.set_queue_rate(0, 500.0);            // 500 Mbps sustained rate

fc.set_queue_burst(0, 16000);           // 16 KB burst capacity

// With 1000-byte packets: first 16 packets arrive instantly (burst),

// then packets are rate-limited to 500 Mbps

## 6.3 Multi-Queue WRR Scheduling

Weighted Round Robin distributes bandwidth proportionally to queue weights.

flow_controller fc = new();

byte unsigned pkt[$];

fc.set_port_rate(10000.0);

fc.set_duration(100us);

// Add 3 queues with weights 4:2:1

fc.add_queue(0, MODEL_RATE);

fc.set_queue_rate(0, 5000.0);

fc.set_queue_weight(0, 4);              // 4/(4+2+1) = 57% bandwidth

fc.add_queue(1, MODEL_RATE);

fc.set_queue_rate(1, 5000.0);

fc.set_queue_weight(1, 2);              // 2/(4+2+1) = 29% bandwidth

fc.add_queue(2, MODEL_RATE);

fc.set_queue_rate(2, 5000.0);

fc.set_queue_weight(2, 1);              // 1/(4+2+1) = 14% bandwidth

fc.set_scheduler_mode(SCHED_WRR);

// Push packets to all queues

pkt = {};

for (int i = 0; i < 1500; i++) pkt.push_back(8'hAA);

for (int i = 0; i < 500; i++) begin

fc.push_packet(0, pkt);

fc.push_packet(1, pkt);

fc.push_packet(2, pkt);

end

fc.start_schedule();

// ... get_next_scheduled loop ...

fc.report();

## 6.4 Strict Priority Scheduling

Higher-priority queues are always served first.

fc.add_queue(0, MODEL_RATE);

fc.set_queue_rate(0, 5000.0);

fc.set_queue_priority(0, 3);            // Highest priority

fc.add_queue(1, MODEL_RATE);

fc.set_queue_rate(1, 3000.0);

fc.set_queue_priority(1, 2);            // Medium priority

fc.add_queue(2, MODEL_RATE);

fc.set_queue_rate(2, 1000.0);

fc.set_queue_priority(2, 1);            // Lowest priority

fc.set_scheduler_mode(SCHED_STRICT_PRIORITY);

// Queue 0 served exclusively until empty, then queue 1, then queue 2.

## 6.5 Mixed SP+WRR Scheduling

Combine SP for critical traffic with WRR for best-effort traffic.

// High-priority control plane traffic (SP)

fc.add_queue(0, MODEL_RATE);

fc.set_queue_rate(0, 1000.0);

fc.set_queue_priority(0, 5);            // prio > 0: SP mode

// Best-effort data plane traffic (WRR, prio = 0)

fc.add_queue(1, MODEL_RATE);

fc.set_queue_rate(1, 3000.0);

fc.set_queue_priority(1, 0);            // prio = 0: WRR mode

fc.set_queue_weight(1, 3);

fc.add_queue(2, MODEL_RATE);

fc.set_queue_rate(2, 3000.0);

fc.set_queue_priority(2, 0);

fc.set_queue_weight(2, 1);

fc.set_scheduler_mode(SCHED_SP_WRR_MIXED);

// Queue 0 always served first (SP).

// When queue 0 is empty, queues 1 and 2 share bandwidth at 3:1 ratio (WRR).

## 6.6 Burst Traffic Pattern

Send N packets back-to-back, then pause, then repeat.

fc.add_queue(0, MODEL_BURST);

fc.set_burst_param(0,

5,              // send_count: 5 packets per burst

10us            // pause_time: 10 us between bursts

);

// Traffic pattern: [pkt pkt pkt pkt pkt] --10us-- [pkt pkt pkt pkt pkt] --10us-- ...

## 6.7 Random Traffic Model

Generate traffic with stochastic inter-packet gaps.

// Uniform random

fc.add_queue(0, MODEL_RANDOM);

fc.set_random_param(0, 2000.0, DIST_UNIFORM);    // 2 Gbps avg, uniform

// Poisson process (memoryless)

fc.add_queue(1, MODEL_RANDOM);

fc.set_random_param(1, 1000.0, DIST_POISSON);    // 1 Gbps avg, poisson

// Gaussian random

fc.add_queue(2, MODEL_RANDOM);

fc.set_random_param(2, 500.0, DIST_NORMAL);      // 500 Mbps avg, normal

## 6.8 Step Rate Profile

Define a multi-segment rate profile that changes over time.

fc.add_queue(0, MODEL_STEP);

// Define rate steps

begin

step_cfg_t steps[$];

step_cfg_t s;

s.duration = 10us;  s.rate_mbps = 1000.0;   // 0-10us:  1 Gbps

steps.push_back(s);

s.duration = 20us;  s.rate_mbps = 5000.0;   // 10-30us: 5 Gbps

steps.push_back(s);

s.duration = 10us;  s.rate_mbps = 500.0;    // 30-40us: 500 Mbps

steps.push_back(s);

fc.set_step_param(0, steps);

end

// To loop the profile continuously:

// step_model sm;

// $cast(sm, queues[0].model);

// sm.loop_enable = 1;

## 6.9 Custom Traffic Model

Create your own traffic model by extending traffic_model_base.

// Define custom model

class my_model extends traffic_model_base;

real base_rate_mbps;

int pkt_count;

function new();

super.new();

model_type = MODEL_CUSTOM;

base_rate_mbps = 1000.0;

pkt_count = 0;

endfunction

virtual function realtime get_interval(int pkt_size, realtime current_time);

real rate;

pkt_count++;

// Increase rate every 10 packets

rate = base_rate_mbps * (1 + pkt_count / 10);

return (pkt_size * 8.0) / (rate * 1_000_000.0) * 1s;

endfunction

virtual function void reset();

pkt_count = 0;

endfunction

virtual function string to_string();

return $sformatf("my_model: base_rate=%.1f Mbps", base_rate_mbps);

endfunction

endclass

// Use custom model

fc.add_queue(0, MODEL_CUSTOM);

begin

my_model m = new();

m.base_rate_mbps = 500.0;

fc.set_queue_model(0, m);

end

## 6.10 Statistics & Monitoring

Access detailed per-queue statistics after simulation.

// After scheduling completes:

queue_stats_t stats;

// Method 1: Print formatted report

fc.report();

// Output:

// === Flow Controller Report ===

// Duration: 100.000 us | Window: 10.000 us | Tolerance: 5.0%

// Queue | Config Rate | Actual Rate | Dev    | Packets | Bytes     | Violations

// 0     | 1000.0 Mbps | 1090.9 Mbps |  9.1%  |      12 |     12000 | 0

// Total: 12 packets, 12000 bytes

// === END ===

// Method 2: Access stats programmatically

stats = fc.get_queue_stats(0);

$display("Queue 0: rate=%.1f Mbps, deviation=%.1f%%, packets=%0d",

stats.actual_rate_mbps, stats.deviation_pct, stats.total_packets);

// Check rate accuracy

if (stats.deviation_pct > 10.0 || stats.deviation_pct < -10.0)

$error("Rate deviation too high: %.1f%%", stats.deviation_pct);

// Check for window violations

if (stats.window_violations > 0)

$warning("Queue 0 had %0d window violations", stats.window_violations);

## 6.11 Complete Test Bench Example

A complete test bench with multi-queue WRR scheduling and statistics verification.

program complete_testbench;

`include "core/flow_controller.sv"

initial begin

flow_controller fc = new();

byte unsigned pkt[$];

int qid;

byte unsigned out_data[$];

realtime send_time;

int q0_count, q1_count, total;

queue_stats_t s0, s1;

// ---- Configuration ----

fc.set_port_rate(10000.0);          // 10 Gbps port

fc.set_duration(200us);             // 200 us simulation

fc.set_debug_level(0);              // 0=quiet, 2=trace

// ---- Queue Setup ----

fc.add_queue(0, MODEL_RATE);

fc.set_queue_rate(0, 3000.0);       // 3 Gbps

fc.set_queue_weight(0, 3);

fc.add_queue(1, MODEL_RATE);

fc.set_queue_rate(1, 2000.0);       // 2 Gbps

fc.set_queue_weight(1, 1);

fc.set_scheduler_mode(SCHED_WRR);

// ---- Packet Generation ----

pkt = {};

for (int i = 0; i < 1500; i++) pkt.push_back(i % 256);

for (int i = 0; i < 1000; i++) begin

fc.push_packet(0, pkt);

fc.push_packet(1, pkt);

end

// ---- Scheduling ----

q0_count = 0;

q1_count = 0;

total = 0;

fc.start_schedule();

while (fc.get_next_scheduled(qid, out_data, send_time)) begin

if (qid == 0) q0_count++;

else q1_count++;

total++;

end

// ---- Results ----

fc.report();

s0 = fc.get_queue_stats(0);

s1 = fc.get_queue_stats(1);

$display("");

$display("Scheduling Results:");

$display("  Total packets sent: %0d", total);

$display("  Queue 0: %0d packets (%.1f%%)", q0_count, 100.0*q0_count/total);

$display("  Queue 1: %0d packets (%.1f%%)", q1_count, 100.0*q1_count/total);

$display("");

$display("Rate Verification:");

$display("  Queue 0: configured=%.1f, actual=%.1f, dev=%.1f%%",

s0.configured_rate_mbps, s0.actual_rate_mbps, s0.deviation_pct);

$display("  Queue 1: configured=%.1f, actual=%.1f, dev=%.1f%%",

s1.configured_rate_mbps, s1.actual_rate_mbps, s1.deviation_pct);

// ---- Assertions ----

if (q0_count <= q1_count)

$error("WRR failed: q0 (weight=3) should send more than q1 (weight=1)");

if (s0.total_bytes != q0_count * 1500)

$error("Stats mismatch: bytes != packets * pkt_size");

$display("\nTest completed successfully.");

end

endprogram

# 7. Test Suite

The project includes 89 test cases across 6 test files:

| Test File | Tests | Coverage |
| --- | --- | --- |
| test_traffic_models.sv | 19 | All 5 models: rate, token bucket, burst, random, step |
| test_queue.sv | 12 | FIFO: push/pop, depth, flush, model assignment, peek |
| test_scheduler.sv | 17 | SP, WRR (3:1, 1:1, 4:2:1), drain, mixed, preemption, reset |
| test_port_shaper.sv | 4 | Token refill, burst exhaustion, wait time |
| test_flow_controller.sv | 29 | Integration: integrity, SP/WRR, rate, stats, burst, duration, monotonicity |
| test_monitor.sv | 8 | Statistics, multi-queue tracking, deviation, debug |

Run all tests:

make test_all

Run individual test:

make test_scheduler    # or test_queue, test_traffic_models, etc.

# 8. Building & Running

## Prerequisites

Synopsys VCS simulator with valid license.

## Environment Setup

export VCS_HOME=/opt/synopsys/vcs/Q-2020.03-SP2-7

export PATH=$VCS_HOME/bin:$PATH

export LM_LICENSE_FILE=30000@<license_server>

## Makefile Targets

| Target | Description |
| --- | --- |
| make test_all | Run all 6 test suites |
| make test_traffic_models | Test traffic models |
| make test_queue | Test queue operations |
| make test_scheduler | Test scheduling algorithms |
| make test_port_shaper | Test port rate limiter |
| make test_flow_controller | Test flow controller integration |
| make test_monitor | Test statistics monitor |
| make clean | Remove build artifacts |

## Compilation Flags

VCS_FLAGS := -full64 -sverilog -timescale=1ns/1ps -f filelist.f +incdir+src

## Custom Test Bench

To compile and run your own test bench:

vcs -full64 -sverilog -timescale=1ns/1ps \

-f filelist.f +incdir+src \

your_testbench.sv -o simv

./simv

## Debug Mode

Enable packet-level tracing:

fc.set_debug_level(2);

// Output: [8000] TX queue=0 pkt_size=1000

//         [16000] TX queue=0 pkt_size=1000